Operation of Singba Light APP K-1000C/K-8000C

一、Singba Light APP

1. Scan the QR code to download. You can scan and download it with wechat

Android system                                     Apple IOS system

2. Click to install the APP

3. Bluetooth Connection (Taking K-8000C as an Example)

3.1 Turn on the Bluetooth of your mobile phone

3.2 Open the APP

3.3 Search for devices, click on the device you need to control, and connect it

3.4 Bluetooth connection successful, enter the operation interface.

二、APP operation

1. Interface

Program broadcast             Dimming test              Write code             "Settings

Play SD card or built-in effects.  Overall dimming of the lamps   DMX lamp coding and testing  System parameter Settings

2. Program broadcast

2.1 Program List: SD card program list or built-in effect program list

2.2 Playback Settings:

Programs: SD card programs or built-in effect programs (22 types)

Speed: 1-16 levels

Chip model: Built-in effects need to be set when playing

2.3 Others

Loop/Non-loop: Click to switch

Self-playback: When the controller has scheduled playback, click to resume scheduled use

Light on/Off: Click to switch, and the light will turn on/off

2.4 Built-in Effects Playback - Channel Selection

For the operation of playing built-in effects without inserting an SD card, it is necessary to switch

between 3 channels or 4 channels

Click "Settings" - > "System Settings" - > "Number of Built-in Effect Channels"

3. Dimming operation

Atmosphere mode     Color temperature mode     Lighting mode   Mixed light mode

3.1 Dimming chip: Select the corresponding chip for the lamp

3.2 Dimming Channel: Set the correct channel for the lamp (depending on the lamp selection, options 1 to 6 are available)

3.3 Switching speed: 1-16 levels

3.4 Channel Color values: The color values of each channel corresponding to the dimming color

3.5 Save: After setting the dimming, click Save. The controller will always remain in the dimming state.

3.6 Dimming channel sequence: Adjustment of the color sequence of the lamps during dimming tests

3.7 Mode Selection

Atmosphere mode: Seven-color dimming

Color temperature mode: Color temperature adjustment (1000K-9700K) adjustment

Lighting mode: Dimming of lighting brightness

Mixed light mode: Channel dimming method

4. Code writing (for DMX512 lamps) and decimal points

DMX coding                   DMX address                 Decimal point function

4.1 DMX coding

1. Chip model: Select the DMX chip for the lamp

2. Coding method: Coding by channel (default)/Coding by the number of lights

3. Initial channels/Number of lamps: The initial number of channels or lamps of the luminaire

4. Interval channels: The number of intervals between lamps

5. Port: Controller write port selection (You can choose to write ALL or select a single port)

6. Writing Code: Click "Start Writing" (it usually takes 10 to 18 seconds)

7. Special coding

Unified address: Write one address for all lamps.

Repeated address writing: For lamps, write the same address repeatedly (the number of repeated lamps = the number of repetitions +1. For example, if 5 lamps repeat the same address, fill in 4 for the number of repetitions).

4.2  DMXAddress

1. Chip model: Select the DMX chip for the lamp

2. Interval channels: The number of intervals between the lamps

3. Port: Controller test port selection (ALL or single selection is available.

A certain port

4. Manual testing: Manually click the "+" or "-"; Or manually enter the number of lights for testing.

Compensation mode: Check the option to continuously send test signals (for some chips)

5. Automatic Testing: Click on the lamp to automatically test from front to back (from 1 to 512)

4.3 Decimal point function

1. Spaced channels

2. Port: (Valid when selecting port decimal points)

3. Lamp serial number: (Valid when selecting point by point)

4. With trajectory/Without trajectory: After the decimal point, whether the lamp remains on.

5. Manual testing/automatic testing

5. "Settings

Click "Settings" to enter the interface.

5.1 System Settings

Setting 1. Set the color sequence for dimming: This is the adjustment of the color sequence of the lamps during the dimming test.

Setting 2.Modbus address: Used when the Modbus address of the controller needs to be modified.

Setting 3. Device Time Correction: For devices with timing (timing) functions, time correction is provided.

Setting 4. Number of built-in effect channels: This is used when channel switching is required during the playback of built-in effects.

Restore factory Settings: Clicking will restore the core, channel and other Settings in the controller to factory Settings.