4G remote control (mobile phone)

I. Hardware Equipment

4G main control and distributed control smartphone

4G data SIM card , network cable and other cables

II. Software Program

The "Smart Cloud Control System APP" software installed on the mobile phone

III. Controller Settings

1. Control and lighting fixtures: Ensure that the wiring is properly connected and that the lighting fixtures operate normally;

2. Main controller and sub-controllers: Connect using network cables (network ports) and test that the lines between each controller node are normal; ( the built-in 4G main controller and sub-controllers can only be connected to the main controller's network port 1 ).

3. Insert a 4G data card into the SIM card slot of the main controller and connect the SIM antenna.

4. 4G switch ( on by default at the factory )

Note 1: For the main control and sub-control, each pair of nodes can be cascaded up to 100 meters using Cat5e or higher specification network cables . For distances exceeding this, switches or fiber optic cables can be added for long-distance transmission.

Note 2: The network cable is crimped using the 568B straight-through method.

IV. Software Settings

1. The "Smart Cloud Control System APP" software installed on the mobile phone.

2. Installation complete. Click the icon shown below to open the software. 3. Open the software — software login interface.

4. Register as a member --

Note 1: Mobile login -- Member: Device Management, Member Management (partial).

Mobile login -- Operator: Device Management.

Note 2: If the password is lost, all three security answers must be used together (all must be correct).

- Log in with your username and password                           6. The software control interface is shown in the following figure:

As shown in the image below:

V. Software Usage

1. Interface Introduction

1 ) Device Management Interface 2) Member Management Interface 3) Device Management Interface

Equipment Management

Click "+" to access the function list.

- smart switch

Click to enter the high-voltage module settings

- Smart Lighting

Click to enter Low Voltage - Controller & Lighting Settings

- Add device

Add main control device

- Remove device

Remove main control device

- Program renaming

Rename the program on the main control SD card

- Log out

A. Adding devices

Enter the master controller's UID number ; set the device name ; click OK to complete "Add Device".

B. Remove device

On the main interface, select the device item and click "Confirm" to complete the "Delete Device" process.

- Program renaming

Change the program name on the SD card and save the changes.

D. Intelligent switch — Main control high-voltage module

Control the high-voltage module (on/off, timed on/off). Once the settings are complete, download them to the controller and restart for them to take effect.

E. Smart Lighting - Main Control

1. Select the device

2. Click to enter — Lighting Configuration

1) Local effect: (SD card effect)

Play program, playback speed, scene control, channel brightness (permanently saved), loop, non-loop, auto-play, save current, restart main control.

Note 1. During channel brightness adjustment, the command will only be sent after you have set the color and click "Set".

Note 2. To save the changed settings, click "Save Current".

2) Built-in effects:

Playback program, IC model, number of channels, playback speed, play, pause, restart master control, dimming (temporary)

Note 1. During the dimming operation, pause the playback of the built-in effects first. After setting the color, click "Settings" for the command to be sent.

3) DMX settings:

Light fixture coding and DMX address testing

4) Timer settings:

Enable/disable remote timer, remote timer settings

- Accessibility

Loop Sync Switch: This switch enables loop playback of programs when GPS is enabled. This function is only applicable to 4G devices with main control version 4.6 and above.

F. Right-click function

Long press the device item to display the right-click menu.

Remove device : Remove device;

Disable : Disable the device;

Enable : Enable disabled devices;

Change Name : Click to enter the device name modification interface and change the device name;

Equipment editing : Assign equipment to project names or to sub-account members;

Set SIM card number : Used to check the usage status of 4G data SIM card;

Member Management

A. Add member (sub-account) B. Delete member

Adding content is the same as the member registration process: 1. Select members.

2. To delete a member, click "Confirm".

C. Freeze members D. Unfreeze members

1. Select members . 1. Select frozen members.

2. Freeze members, 2. Unfreeze members ,

Display Status - Disabled (Account actions for this member are invalid) Display Status - Enabled (Account actions for this member are valid)

E. Right-click function

Delete : Delete a member;

Deactivate : Member permissions are deactivated;

Enable : Re-enables disabled members;

Bluetooth debugging

A. Turn on your phone's Bluetooth function to debug the controller at close range (within 1-10m).

B. Click "Software Login Program" ---- "Bluetooth Debugging" C. Select the controller (Bluetooth number) to be debugged.

D. Access the controller's "Lighting Configuration" interface.

The operation method for configuring lights is the same as that for configuring lights via 4G cloud.