4G Remote Control PC Version (v3.0)

I. Hardware Equipment

Main control and sub-control computers (must be connected to the network)

4G router , network cable and other cables

External 4G router connection diagram

II. Software Program

Intelligent Cloud Control System (PC Version)

III. Controller Settings

1. Control and lighting fixtures: Ensure that the wiring is properly connected and that the lighting fixtures operate normally;

2. Main controller and sub-controllers: Connect using network cables (network ports) and test that the wiring between each controller node is normal;

3. Insert a 4G data card into the SIM card slot of the main controller and connect the SIM antenna.

4. 4G switch ( on by default at the factory )

Click to switch--- On/Off

Note 1: For the main control and sub-control, each pair of nodes can be cascaded up to 100 meters using Cat5e or higher specification network cables . For distances exceeding this, switches or fiber optic cables can be added for long-distance transmission.

Note 2: The network cable is crimped using the 568B straight-through method.

IV. Software Settings

1. Install and extract the "Smart Cloud Control System" software (version 3.0) on your computer.

2. Once installation is complete, click the icon shown in the image below to open the software.

3. Open the software — software login interface

4. Register as a member

5. Enter your account to log in

6. Software control interface

- Software usage

Select "Device Management" -> "Smart Lighting" and double-click the project name to open the project equipment list.

1. Interface Introduction

Equipment Management

- Function Area Introduction

Switch Configuration: High-voltage module parameter and function settings

Lighting configuration: Low-voltage (controller) parameter and function settings

Add Device: Add Main Controller

Deleting a device: Deleting the main controller

Program renaming: Edit the main program name

A. Adding devices

Enter the controller's UID number ; set the device name ; click OK to complete "Add Device".

Note: The device UID can be identified by the controller or read by the controller.

B. Remove device

On the main interface, select the device item; then click OK to complete the "Delete Device" process.

- Program renaming

Naming programs on the SD card in the main control unit

- In the Device Management interface, select the main control device in the Device List that needs to be renamed.

2. Click "Rename Program" to enter the interface.

3. Change the program name as required, for example, as shown in the image below.

Note: Clicking "Load Defaults" will restore the default state (the program is unnamed).

4. Click "Confirm" to complete the settings.

5. Go to "Lighting Configuration" -- "Local Effects" -- play the program to view.

D. Lighting Configuration - Main Control

1. Select Devices - Check the boxes (multiple selections or single selections are allowed).

2. Click to enter — Lighting Configuration

1) Local effect:

Playback program and playback speed: Switch between programs and playback speed.

Scene control: play, pause, loop, no loop, auto play, save current settings, restart master control.

Channel brightness: Adjust the brightness.

Note 1. After completing the channel brightness adjustment settings, click "OK" for the command to be sent.

2) Built-in effects:

Program switching, IC selection, channel selection, speed setting, scene control, dimming (temporary), restarting the master control.

Note 1. During dimming, pause the built-in effects first, and after setting the color, click "Set Channel Brightness" for the command to be sent.

3) DMX settings:

Light fixture coding and DMX address testing.

4) Timer settings:

Enable/disable remote timer and remote timer settings.

- Accessibility features (password required for operation - contact sales)

Unencrypted: Remove encryption.

Level 1 encryption: The controller needs to be online (if the controller disconnects from 4G, it will automatically lock after 20 minutes).

Level 2 encryption: The controller directly locks the device.

Cyclic Synchronization On/Off: Cyclicly plays the switch while GPS is on ( this function is applicable to version 4.6 and above ).

Factory reset: Restores the device to its factory default settings.

E. Right-click function

1) In the device list interface, click on the device; then right-click to bring up the context menu;

Menu Directory

- Check Status: Used to check the status of high-voltage electricity;

- Sub-controller status: Check if the sub-controller is online;

3. Switch Configuration: Click to enter the high-voltage configuration interface;

4. Lighting Configuration: Click to enter the lighting configuration interface;

- Enable: Enable the device;

- Disable: Disable the device;

1.) After the device is disabled, the device configuration will indicate that it has been disabled , as shown in the following figure (computer terminal account interface).

2. ) After the device is disabled, the device file download program will be unable to proceed, as shown in the image below (computer terminal account interface).

- GPS Enabled: Enables device GPS synchronization;

- GPS off: Disable device GPS synchronization.

- Add a device: Add a device;

- Remove device: Remove device;

11. Change Name: Change the device name;

12. Equipment Editing: Create a new project name; assign equipment to the project name or to sub-account members;

13. Set SIM card number: Enter your SIM card number to check your 4G data plan usage;

4G SIM card data usage query interface

Download file

A. Download Files - Interface Screenshots (Lights & Switches and Cloud Download)

B. Operation Procedure (Lights & Switches) Note: Applicable to 4G devices with main control version 4.6 or lower.

1. Select equipment

2. Click - "Add Task"

3. Locate the file and add it.

1) Locate the file (with the .zbin extension).

The software generates files that can be transferred via 4G.

Note 1: How .zbin files are generated.

- Outputting Results Files ---- Exporting bin files in T_group

- After completing the group settings, check the box to export the zbin file, as shown in the image below.

3) Generate file — folder—group X.zbin (X is the number of groups) as shown in the image below.

4) Open the group folder — the file with the .zbin extension is the downloadable file from 4G Cloud.

4. Download files

Click to start downloading . The download is complete when the file is 100% downloaded.

Note: After the controller completes the download, it will decompress the files and then automatically restart and refresh the files. (There will be a period of offline status, after which the program will come online again.)

C. Cloud Download (Applicable to 4G main control systems of version 4.6 and above. Devices with UIDs starting with XB21 are automatically categorized into this directory. Cloud download allows for uploading, downloading, or deleting individual programs.)

Delete program: Delete the selected program;

Format: Format the main controller SD card;

Query capacity: Query the capacity of the main controller SD card;

Version Query: Query the main control version;

Cancel Download: Cancel the program that is currently downloading;

1. Select equipment

2. Click - "Add Task"

Public catalog: Multiple devices use the same program files;

Device catalog: Programs are only used on the current device;

3. Click "Add Files"

1) Locate the file (with the .xbin extension).

The software generates files that can be transferred via 4G.

Note 1: How .xbin files are generated.

the effect file----T_Group Export bin

After completing the group settings, check the "Export xbin file" option, as shown in the image below.

Generate a file named X.xbin (where X is the number of groups) from a folder in the command prompt. See the image below.

d. Open the xbin folder in the group; the file with the .xbin extension is the 4G cloud download result file.

- Upload files to the cloud server; you can upload single files or select all files. Once uploaded, you can edit and change the section numbers .

- Once the program is downloaded , select the file, click the download command, and you can view the progress in the progress list (click details to view the download progress).

Member Management

A. Add members (add sub-accounts)

The process of adding content is the same as the membership registration process.

B. Delete member

1. Select members

2. To delete a member, click "Confirm".

C. Freeze members

1. Select members

2. Freeze a member's account; the status will then be displayed as "Disabled" (account actions for this member are invalid).

D. Unfreezing members

1. Select the frozen members.

2. Unfreeze the member, and the status will be displayed as " Enabled" (the member's account operations are valid).